General Specifications

Carver Homestead West Condominiums II

1132 330th Street

Rhodes, IA 50234

The following document is an outline specification and summary of work that Jensen Design Build, Inc. will perform for the noted residence/project.  This general specification and summary is not intended to be all-inclusive, but specifies the major components of the project and implies the level of quality expected of the minor components not listed.  This document does not supersede any building code or permit requirements.  Materials of equal value, as determined by Jensen Design Build, Inc., may be substituted for materials specified.

General Description

A group of luxury condominium units, with two bedrooms, 2 or 2-1/2 baths, located on a small hill overlooking Hole No. 3 of The Harvester Tournament Course

Finished living space: 1,106 s.f., 1,079 s.f., 1,225 s.f. or 1,336 s.f. (depends on unit)

Structure

Poured concrete walls (8” nominal thickness)

Conventional wood framing system with trussed roof

2 x 4 exterior and interior walls, 16” o.c.

2 x 10 floor system, 16” o.c.

3/4” tongue and groove floor decking

1/2” OSB roof sheathing w/clips (24” o.c. truss spacing)

Four-inch concrete slab floor with four-inch granular fill

Exterior

1/2” Dow Styrofoam exterior sheathing (taped joints) with 1/2” wood corners for wind bracing

Brick/cultured stone veneer and HardiPlank cement board siding with painted cedar accents (as per plans)

Wood fascia and soffits

Seamless aluminum gutters and downspouts, 32 ga.

GAF Timberline Natural Shadow, 25-year asphalt shingles (or equivalent) with 15 lb. felt paper

Deck/Porch

Treated wood post and beam structure with cedar decking and cedar railings

Doors and Windows

Pella Pro-Line double-hung windows (clad) with muntins

Pella patio doors (clad)

Pella steel exterior entry doors

Insulation

R-13 fiberglass insulation in exterior walls with 4 mil. poly moisture barrier

R-40 cellulose insulation in attic

Drywall/Painting

1/2” drywall (5/8” on trusses) with knock-down texture on ceilings and orange peel texture on walls

Sherwin-Williams or Iowa Paint throughout: white eggshell, on all ceilings and walls

Interior trim to be stained, sealed and urethaned

All exterior steel doors painted white (interior and exterior)

Cabinets and Countertops

Omega Dynasty cabinets in kitchen and bathrooms

Laminate countertops with 4” backsplash and no-drip edge in kitchen

Tall vanities in master bathroom

Cultured marble tops in all bathrooms

¼” plate glass mirrors above all bathroom vanities, with chrome channel and clips

2 towel bars and 1 paper holder for each full bath; 1 towel bar and 1 paper holder for ½ bath

Trim

Oak interior trim, colonial or mission style

Oak handrails, balusters and posts

Oak 2-panel or 6-panel, solid-core interior doors (flush panel for all bi-fold doors)

Skirt board along stairway (Unit 4)

Schlage door handles and locks

Flooring

Hardwood: ¾” thick by 2-1/4” wide, No. 1 oak

Tile: Daltile Calabria Biscotti 12x12

Carpet: Shaw Huntland

(Location of all types as per plans)

Mechanical Systems

Lennox high-efficiency 92% gas furnace with 13-seer central air-conditioning unit

200-amp underground electrical service and breaker panel

Generous electrical outlets/switches throughout interior; 1 or 2 weatherproof exterior receptacles

Telephone and cable jacks: kitchen, living room, all bedrooms

Kichler light fixtures, Quorum fan kits

Fan/light exhaust fans for all bathrooms

Doorbell for front door

50-gallon high recovery gas water heater

Delta faucets

Mansfield water closets

Mansfield pedestal sink (Unit 4: ½ bath)

Aquaglass fiberglass tub/shower and shower units in bathrooms (as per plans)

Americast kitchen sink

ISE Badger V disposal (kitchen)

1 or 2 exterior hose bibs

Appliances

Whirlpool dishwasher (Model #DU920PFGQ)

Whirlpool glass-top electric range (Model #RF386PXGQ)

Whirlpool over-the-range microwave with exhaust hood (Model #MH6130XEQ)

Refrigerator (model not yet determined)

Washer and dryer (models not yet determined)

Hookups for washer and electric outlet for dryer

Ice maker water lines for refrigerator

Specialty Items

Royal 36” gas fireplace (Model #R5500TR), with oak mantel, tile surround and flush tile hearth (Daltile Calabria Alfredo 8x10 and 12x12)

Closet Maid custom wire closet systems (as per plans)

Site

Four-inch concrete parking areas with #4 reinforcing bar (as per plans)

Four-inch sidewalks (as per plans)

Complete landscaped yard with necessary retaining walls

Sod and/or seed, depending on size and slope of area

Warranty

Jensen Design Build, Inc. Home Warranty

West Condominium I Association Dues

Exterior Maintenance

$40/month

Common Utilities

$40/month

Future Maintenance

$30/month

Insurance


$55/month

Property Taxes


$135 to $185/month

Utilities



$60 to $80/month

TOTAL



$360 to $430/month

The Harvester Community Association Dues

The overall community dues cover homestead entrance areas, street lights, future parks and any improvements that benefit the entire community. Currently, there are no dues in this category, but they will be established in the future.

Note: All costs listed above are approximate and are subject to change.

Revised: February 24, 2004
The Harvester Community

104 Foster Drive

Rhodes, IA 50234

(641) 227-4653

www.harvestergolf.com

